REQUEST FOR BID

Contact for Information:

| Client: James Reilly Property: 2304 West Blvd. Holt, MI | SCOTT DOERFLER INGHAM COUNTY LAND BANK |
| --- | --- |
|  | (517) 267-5221 ext. 2380 |

| Bids Due: | April 30, 2026 |
| --- | --- |

2. GENERAL BUILDING PERMIT

$_                   __ Spec:26.2041   GENERAL BUILDING PERMIT

General Building Permit

Cost of building permit for general work as specified. Please note that on projects involving federal funds, the building permit fee will be based on total value of all the construction work. In addition, each trade will also be required to pull permits for their associated line items. Your price should reflect this fee structure.

Note: Contractor to supply Ingham County Land Bank with building permit receipt when submitting for draw and finalized building permit at the completion of job.

5. DUMPSTER SERVICE

$_                   __ Spec:04.009   DUMPSTER SERVICE

Dumpster Service

Furnish dumpster service for construction on site.

6. PORT-A-JOHN ON SITE

$_                   __ Spec:02.001   GENERAL (TYPE IN ITEM TOPIC HERE)

Port-a-john on site

Furnish a port-a-john on site, in secure location, from beginning of project until bathroom facilities are available on site.

Location: On project site or as close as allowable.

51. WALL AND FLOORING REMOVAL (ATTIC AREA)

$_                   __ Spec:20.007   WALL AND FLOORING REMOVAL

Wall and Flooring Removal (Attic Area)

Remove and dispose of knee walls, framing, sheathing, trim and insulation (if applicable). Remove all ceiling surface materials and insulation to expose roof rafters.  Remove flooring material complete to expose ceiling joists.  Work to be done to ensure proper structural integrity and meet all current code requirements.

Location: Attic wings only

55. FIBERGLASS INSULATION

$_                   __ Spec:19.005   FIBERGLASS INSULATION

Fiberglass Insulation

Furnish and install new fiberglass building insulation blankets between existing ceiling joists as manufactured by Owens - Corning Fiberglass Corp., Toledo, Ohio, or approved equal. The thermal resistance (R-Value) of the insulation blanket shall be R-19 for bonds.  Insulation shall be installed in strict accordance with the manufacturers written instructions. Work to meet all current code requirements.

Location: Entire attic

*Note – Furnish and install new ½” subflooring complete, to attic wings as needed.

57. SEALING (AIR INFILTRATION)

$_                   __ Spec:19.009   SEALING (AIR INFLITRATION)

Sealing (Air Infiltration)

Insulate around all windows, doors, outlets, switches and all other penetrations to the exterior walls. Caulk or insulate rim board cavities. Use caulk backer at large openings. Furnish and install approved tape to seal all exposed heating duct and return air seams and boots. Prior to finishing the exterior of the house, install paintable silicone caulk at all trim where wood trim and wood siding meet. Prior to interior painting, install new paintable silicone caulk where all interior trim and plaster /gypsum meet, at window trim, door trim and base trim.

Location: Attic Area Only

58. WALL REPAIR

$_                   __ Spec:20.007   WALL REPAIR

Wall Repair

Replace loose, damaged, and deteriorated wall materials with new gypsum drywall fastened securely to structure. Tape and sand joints to provide surface even with existing adjacent surfaces.  Furnish and apply primer/paint to match existing walls.

Location: Existing furnace location - Hallway

59. RE-DRYWALL DRYWALL-WALL AND CEILING/BATH

$_                   __ Spec:20.002   GYPSUM DRYWALL-WALL AND CEILING

Re-Drywall-Wall and Ceiling/Bath

Remove and dispose of all drywall on bathroom walls and ceiling complete. Bathroom to be stripped to existing wall and ceiling studs.  Furnish and install R19 fiberglass insulation to exterior wall.  Furnish and install new 1/2-inch gypsum drywall, as manufactured by "US GYPSUM", or approved equal to all wall and ceiling surfaces. Fastened securely to structure per manufacturers specifications. Tape and sand joints to provide an even finish.  Provide new metal corner beads at outside corners.

Location: Bathroom

*Note – Furnish and install Greenboard to shower stall walls as needed.

65. PAINTING CEILING AND WALLS

$_                   __ Spec:28.002   PAINTING CEILING AND WALLS

Painting Ceiling and Walls

Prepare and paint wall and ceiling surfaces, trim, moulding, hand rails, doors, door frames, window frames, and window stools with "Sherwin Williams ProMar 200 Zero VOC Interior Latex, B20-2600 Series".  Provide "Sherwin Williams ProMar 200 Zero VOC Interior Latex Primer" coat on new surfaces. Final coat to be semi-gloss on ceiling in bath. Provide eggshell finish on walls. Verify colors with Ingham County Land Bank selection sheet.

Location: Bathroom

70. NEW TRIM

$_                   __ Spec:20.014   NEW WOOD TRIM

New Trim

Remove and dispose of existing base moulding, and door completely. Furnish and install new MDF "Colonial" style #412 4 1/4-inch base and #411 3-inch door casing. Fill holes from installation and caulk to provide a smooth and even surface ready for paint.

Location: Bathroom

Notes:

1)  Use a paintable pine in bathrooms, NOT MDF.

73. WOOD SHOE MOLDING

$_                   __ Spec:02.001   GENERAL (TYPE IN ITEM TOPIC HERE)

Wood Shoe Molding

Remove and dispose of all existing shoe molding. Furnish and install new 1/2-inch x 3/4-inch wood shoe molding to match base molding.

Location: Bathroom

Note:  3/4-inch quarter round can be used if conditions require. Scribe molding will NOT be allowed.

75. KITCHEN CLOSET SHELVES

$_                   __ Spec:20.011   CLOSET SHELVES/RODS

Kitchen Closet Shelves

Remove shelves and accessories completely to provide access to hot water heater.

Location: Kitchen Closet

76. INTERIOR DOOR, PREHUNG

$_                   __ Spec:15.012   INTERIOR DOOR, PREHUNG

Interior Door, Prehung

Remove and dispose of door and jamb and related material completely. Furnish and install new interior hollow core, pre-hung door with six (6) panels as manufactured by "Colonist" or approved equal. Installation to include three (3) hinges, hardware and new passage lockset for a complete installation. Bathroom shall have privacy lockset.

Location: Bathroom

*Note – Lead detected on hallway doorway trim.

78. HOME HARDWARE

$_                   __ Spec:20.013   HOME HARDWARE

Home Hardware

Remove and dispose of existing hardware throughout the bathroom. Furnish and install two (2) new towel bars, one (1) toilet paper holders, shower curtain rod with mounting hardware (no tension rods) in bathroom. Furnish and install new solid hinge and wall bumps, 1/4-inch mirrors, medicine cabinet.

Location: Bathroom

*Note - Furnish and install grab bar directly behind new water closet installation.

79. VANITY

$_                   __ Spec:24.004   CABINETS, WALL AND VANITY

Vanity

Remove and dispose of vanity cabinet, wall cabinet and related material completely. Furnish and install new vanity cabinet as manufactured by AristoKraft or approved equal. Furnish and install matching crown molding, fillers, scribe molding and all necessary accessories. Fasten securely to structure. Verify style and finish with Ingham County Land Bank selection sheet.

Location: Bathroom

85. VINYL FLOORING

$_                   __ Spec:26.001   VINYL FLOORING

Vinyl Flooring

Remove and dispose of existing floor covering. Replace subfloor to provide a smooth, even and sound surface to receive new materials. Furnish and install new underlayment and new sheet vinyl floor covering as manufactured by Congoleum "AirStep Plus" style or approved equal. Install using the fully adhered method using manufacturer recommended adhesive. Where wood base exists, provide shoe moulding where existing wood base and new floor covering meet. Provide finish to match existing wood base. Fasten flooring and base to structure with waterproof adhesive. Undercut all doors to provide a minimum of 1/4 inch clearance above finished floor. Verify color and style with Ingham County Land Bank selection sheet.

Location: Bathroom

*Note – Remove existing subfloor complete.  Remove all damaged floor joists complete.  Furnish and install new floor joists as needed.  Refasten floor joist bracing as needed to meet all applicable local codes.

87. PERMIT FOR PLUMBING CONTRACT

$_                   __ Spec:26.2044   PERMIT FOR PLUMBING CONTRACT

Permit for Plumbing Contract

Cost of permit for Plumbing contract.

Note: Supply Ingham County Land bank with plumbing permit receipt when submitting for draw and finalized permit at the end of job.

88. WATER HEATER-DIRECT VENT

$_                   __ Spec:16.033   WATER HEATER-ATMOSPHERIC VENT

Water Heater-Direct Vent

Remove and dispose of existing water heater and related materials completely. Furnish and install new 40-gallon, gas fired, direct vent water heater, Rheem or approved equal with a minimum 6-year warranty on tank and parts. Water heater must be Energy Star rated; have an energy factor of 0.67 or better; and must have a recovery rate of 79% or better. Provide final hook-ups and venting as required for a complete installation and to meet all current code requirements.

Location: Kitchen Closet Area

89. PEX SUPPLY PIPING TO FIRST FLOOR BATHROOM

$_                   __ Spec:16.000   PEX SUPPLY PIPING TO FIRST FLOOR

PEX Supply Piping to First Floor Bathroom

Remove and dispose of horizontal and vertical hot and cold-water supply piping in bathroom and up to first floor plumbing fixtures. Furnish and install new PEX-A piping and fittings required to provide a complete supply piping system. Install to meet manufacturers written instructions using manufacturer recommended fittings, support hangars and installation methods.  Include new shut-off valves at each plumbing fixture.  Provide braided stainless-steel feed lines at exposed piping areas. DO NOT install PEX piping in an area of direct sunlight. Install so manufacturer’s warranty applies.

Location: Bathroom

92. REPLACE SANITARY PIPING AND TRAP

$_                   __ Spec:16.055   REPLACE SANITARY PIPING AND TRAP

Replace Sanitary Piping and Trap

Replace sanitary piping with new sanitary piping and new sanitary trap including vent. Work to meet all current code requirements.

Location: Bathroom

98. FIBERGLASS SHOWER STALL

$_                   __ Spec:16.060   FIBERGLASS SHOWER STALL

Fiberglass Shower Stall

Remove existing tub and surrounding walls complete. Furnish and install new 34-inch x 60-inch x 72-inch fiberglass shower stall as manufactured by Aquatic, Model #160344 PSL-WHHD (Home Depot) or approved equal. Provide new supply and sanitary piping, new faucet, valves, handles and showerhead as manufactured by MOEN, model 2352 with pressure balance valve or approved equal, for a complete installation. Furnish and install all necessary supply and sanitary piping. Work to meet all current code requirements.

Location: Bathroom

Note* - Furnish and install grab bars to new shower stall walls (three).  Secure properly with blocking as needed.  See selection sheet.

99. REPLACE WATER CLOSET

$_                   __ Spec:16.019   REPLACE WATER CLOSET

Replace Water Closet

Remove and dispose of water closet completely. Furnish and install new white, vitreous china water closet (replace with same color where color water closet is existing unless otherwise specified under location.) as manufactured by Sterling "Rockton Dual Force Series", STE 402021 bowl and STE 402022 tank, or approved equal. Include stainless steel braided supply line and new wax ring, as required for a proper installation. Include new Church "Easy Clean" #540EC seat and bolt covers. Furnish and install all necessary supply and sanitary piping. Work to meet all current code requirements.

Location: Bathroom

*Note – Water closet height 21” or greater.

104. CULTURED MARBLE TOP (30”)

$_                   __ Spec:16.014   CULTURED MARBLE TOP

Cultured Marble Top (30”)

Remove and dispose of lavatory completely. Furnish and install new cultured marble vanity top with integrated recessed oval sink INCLUDING OVERFLOW, as manufactured by "U.S. Marble Countertop" or approved equal, to include new fixtures, valves, fittings, pop-up assembly and faucet as manufactured by MOEN, model L4635 or L4625, or approved equal, for a complete installation. Furnish and install all necessary supply and sanitary piping. Work to meet all current code requirements. Verify color of top with Ingham County Land Bank selection sheet.

Location: Bathroom

106. PERMIT FOR MECHANICAL CONTRACT

$_                   __ Spec:26.2045   PERMIT FOR MECHANICAL CONTRACT

Permit for Mechanical Contract

Cost for permit for Mechanical Contract.

Note: Supply Ingham County Land bank with mechanical permit receipt when submitting for draw and finalized mechanical permit at the end of job.

107.  95% EFFICIENT FURNACE

$_                   __ Spec:17.011   95% EFFICIENT FURNACE

95% Efficient Furnace

Remove and dispose of wall furnace and related equipment completely. Furnish and install a new gas fired Direct Vent high efficiency furnace with a minimum efficiency of 95%. New furnace (Goodman or approved equal) to be AGA Approved and have a minimum 20-year warranty on the heat exchanger. Balance system upon completion of installation. Connect new furnace with new supply and return air plenums and ducts. Install new grills and registers in ceilings. Contractors will be responsible for installing new ductwork for proper distribution to habitable rooms. Contractor to conceal new exposed ductwork when it cannot readily be concealed in existing surfaces. Furnish and install new Energy Star Compliant 5-2 Day programmable electronic thermostat as manufactured by Honeywell, model # TH4210D1005 or approved equal. Contractor will be responsible for proper venting of new furnace. Work to meet all current code requirements.

Location: Attic Area

Notes - Furnish and install new, clean furnace filter at the end of job.

111. VENT FOR BATH FAN

$_                   __ Spec:17.005   VENT FOR BATH FAN

Vent for Bath Fan

Furnish and install new rigid metal vent required to exhaust bathroom ventilation fan/light combination to daylight as required by current code. In unconditioned areas, provide insulation at vent pipe. Coordinate work with Electrical Contractor installing fan/light unit.

Location: Bathroom

112. REPAIR GAS PIPING

$_                   __ Spec:17.022   REPAIR GAS PIPING

Repair Gas Piping

Repair gas piping as required to meet all current code requirements.  (New furnace and hot water heater)

Location: Entire house

114. PERMIT FOR ELECTRICAL CONTRACT

$_                   __ Spec:26.2046   PERMIT FOR ELECTRICAL CONTRACT

Permit for Electrical Contract

Cost of permit for electrical contract.

Note: Supply Ingham County Land Bank with electrical permit receipt when submitted for draw and finalized permit at the end of job.

116. REWIRE

$_                   __ Spec:18.058   REWIRE

Rewire

Remove and dispose of outlets, switches and plates completely. Furnish and install new outlets, switches and plates. Include remove and dispose of all accessible wiring back to panel.  Furnish and install new wiring. Provide 3-way switches for light fixtures in hallways and stairways. Furnish and install outlets to serve water heater, range, microwave, refrigerator, dishwasher, garbage disposal, washer and dryer, furnace and air conditioner, where necessary. Include all GFCI outlets and wiring back to panel as required by current code. Add grounded outlets to front and rear exterior of house and a doorbell at the front entrance. Furnish and install new 120-volt combination smoke detector/carbon monoxide detectors with battery back-up conforming with the current Michigan residential code, section 1210. Install according to manufacturer’s written instructions and specifications. Work to meet all current code requirements.

Location: Entire house

117. CORRECT AND UPDATE WIRING ISSUES

$_                   __ Spec:02.001   GENERAL (TYPE IN ITEM TOPIC HERE)

Correct and Update Wiring Issues

1)   Replace all damaged or unsightly switches, outlets and covers. Color to be white.

2)   Test and identify defective and ungrounded wiring at all switches, outlets and fixtures.

3)   Furnish and install new wiring where wire is deteriorated, unsafe per NEC.

4)   Furnish and install new GFCI outlets where required by code.

5)   Where needed, furnish and install new dedicated circuits for range, a/c, water heater, sump pump, dishwasher, refrigerator, garbage disposal and smoke detectors per NEC.

6)   Where needed, furnish and install dedicated circuits for kitchen.

7)   Install wiring and switches as needed for interior and exterior light fixtures.

8)  Furnish and install new LED bulbs in all fixtures.

9)  Furnish and install GFCI protected outlets at exterior front and rear of house.

10)  All new receptacles to be tamper proof.

11) Furnish and properly install new combo carbon monoxide detector/smoke detector 120 volt with battery back-up. Install according to manufacturer’s installation instructions and local code requirements.

118. 100 AMP SERVICE

$_                   __ Spec:18.039   100 AMP SERVICE

100 AMP Service

Remove and dispose of electrical service equipment and add on panels. Furnish and install new 100 AMP electrical service and circuit breaker panel. Provide a separate appliance circuit for kitchen countertop outlet. When required include new mast, meter socket and meter. Label each circuit at electrical panel. Check and balance total electrical system. Installation of new service shall meet the requirements of the most current edition of the National Electric Code and the most current edition of the adopted Delhi Township Uniform Housing Code

Location: Closet

*Note – Existing panel located in northeast bedroom closet

122. HOME STANDARD BATH VENT

$_                   __ Spec:18.044   BATH VENT AND LIGHT

Home Standard Bath Vent

Furnish and install new bath fan that provides continuous air exchange sized and balanced by mechanical contractor (calculations to be provided) and also have a >80 CFM boost function fan with light switched either at the entrance to the room or with an adjustable time delay function. Additional fan must have a >80 CFM boost function fan with light switch.

Location: Bathroom

123. LIGHT FIXTURES

$_                   __ Spec:18.023   CEILING MOUNTED LIGHT FIXTURE

Light Fixture

Remove and dispose of all light fixtures. Furnish and install new wall mounted light fixture with LED bulbs. Work to meet all current code requirements. See Ingham County Land Bank selection sheet.

Location: Bathroom – Above vanity

33. Cellar Window

$_                   __ Spec:15.032   Cellar Window

Cellar Window

Remove and dispose of existing crawl space window. Furnish and install new crawlspace window with frame and tempered glass complete.  Secure properly.  Furnish and install new peastone fill.  Include window well cover.

Location: Basement, in existing location.

86. CLEARANCE CLEANING

$____________Spec:26.2005   CLEARANCE CLEANING

Clearance Cleaning

After all work is complete, clean all areas of the house as required to eliminate lead dust and to obtain final clearance for lead based paint hazards. Clean using methods approved by HUD and the EPA.

Location: Trim on entry doors.

Note: Ingham County Land Bank will pay for two (2) lead clearance tests, interim and final. All additional or failed tests must be paid to the Ingham County Land Bank in advance of ordering additional tests.

122. Recycling

$_                   __ Spec:

Recycling

Recycling of all construction materials as needed.  Properly dispose of all left over unused construction materials from land-fill as needed.  As we moved towards a more circular environment, promote best practices in recycling and repurposing leftover construction material, by reducing the amount of materials to landfills.  This includes all trades.  Structural, Electrical, Plumbing and HVAC

Additional Notes:

At the conclusion of Construction:

All ducts are to be cleaned of any debris & all floor drains, sump crocks and clean-outs are to be cleaned of any debris

NOTE: Structural, Electrical, Plumbing and HAVC contractors to make all connections per Ingham County Land Bank selection sheets and specifications. All work to be done per local codes.

Contractor will be responsible to clean up the site as needed to have a safe environment during construction phases.

Bid Total $________________________