REQUEST FOR BID

Contact for Information:

| Client: Marie Brawn Property: 621 S. Lansing Rd | SCOTT DOERFLER INGHAM COUNTY LAND BANK |
| --- | --- |
| Mason, MI 48854 | (517) 267-5221 ext. 2380 |

| Bids Due: |  |
| --- | --- |

2. GENERAL BUILDING PERMIT

$____________Spec:26.2041   GENERAL BUILDING PERMIT

General Building Permit

Cost of building permit for general work as specified. Please note that on projects involving federal funds, the building permit fee will be based on total value of all the construction work. In addition, each trade will also be required to pull permits for their associated line items. Your price should reflect this fee structure.

Note: Contractor to supply Ingham County Land Bank with building permit receipt when submitting for draw and finalized building permit at the completion of job.

5. DUMPSTER SERVICE

$____________Spec:04.009   DUMPSTER SERVICE

Dumpster Service

Furnish dumpster service for construction on site.

18. TUCKPOINT FOUNDATION WALLs

$____________Spec:07.003   TUCKPOINT FOUNDATION WALLs

Tuckpoint Foundation Walls

Remove cracked, broken, and deteriorated mortar. Remove foam insulation from fieldstone foundation system.  Tuckpoint to provide a sound and watertight wall. Tool joints to match existing adjacent joints. Match color of adjacent mortar.

Location: Exterior and interior of entire house.

35. STEEL DOOR, HALF CIRCLE LIGHT W/ LOCK

$____________Spec:15.004   STEEL DOOR, HALF CIRCLE LIGHT W/ LOCK

Steel Door, Half Circle Light W/ Lock

Remove and dispose of front and rear entry doors and related material completely.  Furnish and install new steel, prehung, insulated doors as manufactured by Therma Tru, "Profiles" SERIES 24-Gauge #255HD.  New steel door to have a minimum 10-year warranty.  New door to have a half circle divided light at the top, and be embossed with four panels.  Installation to include new peep site, weather-stripping, threshold, and lockset/deadbolt tandem keyed to each other. Provide "GRADE 2" deadbolt and "GRADE 3" cylindrical lockset, or approved equal. UPON COMPLETION ALL EXTERIOR LOCKSETS/DEADBOLTS INCLUDING GARAGE WILL OPEN WITH ONE KEY.  New door to have a sill cover or adequately protect door sill during construction. New door to meet all current code requirements.

Location: Front (1) entry door, in existing location.  Rear (1) entry door, in existing location.

Note – Lead detected on trim at door locations

43. ARCHITECTURAL SHINGLES

$____________Spec:11.020   ARCHITECTURAL SHINGLES

Architectural Shingles

Remove and dispose of shingles and related material. Furnish and install new 15 pound felt paper over entire roof. Furnish and install new ice and water shield at roof edges from the eave edges up the roof to at least two feet past the exterior wall, and full roll width in valleys. Furnish and install Architectural self-sealing, asphalt roofing shingles with a class "A" fire rating and a 30-year warranty, as manufactured by CertainTeed "LANDMARK" Series, or approve equal. Fasten new shingles securely to structure as recommended by the shingle manufacturer. Install new aluminum drip edge at roof edges. Ensure that sound weathertight flashing is installed according to shingle manufactures written instructions, at valleys, junctions of roof and wall surfaces and at roof penetrations, include the B-vent. Complete roofing to be free of scratched, dented or split shingles for a weathertight surface. Furnish and install new attic ventilation as per manufactures written requirements.  Furnish Land Bank with a written warranty from the manufacturer for the product installed for a minimum of 30 years. Verify shingle color with Ingham County Land Bank selection sheet.

Location: House ONLY

Note – Color and style to be Weathered Wood or approved equal.

Notes:

1)  Include a separate per sheet, installed price for 7/16-inch OSB sheathing for house.

2)  Include a separate full roof re-deck with 7/16-inch OSB sheathing.

42. FLAT ROOF SYSTEM

$____________Spec:11.020   FLAT ROOF SYSTEM

Flat Roof System

Remove and dispose of existing rolled roofing complete. Furnish and install new PVC roof system.   Furnish and install new ice and water shield at roof edges from the eave edges up the roof to at least two feet past the exterior wall, and full roll width in valleys as needed. Install new aluminum drip edge at roof edges. Ensure that sound weathertight flashing is installed according to roofing manufactures written instructions, at valleys, junctions of roof and wall surfaces and at roof penetrations, include the B-vent. Complete roofing to be free of scratched, dented or split shingles for a weathertight surface. Furnish Land Bank with a written warranty from the manufacturer for the product installed for a minimum of 10 years. Color to be white.

Location: Existing Flat roof system

Notes:

1)  Include a separate per sheet, installed price for 7/16-inch OSB sheathing for house.

2)  Include a separate full roof re-deck with 7/16-inch OSB sheathing.

44. SIDING AND TRIM REPAIR

$____________Spec:13.002   SIDING AND TRIM REPAIR

Siding and Trim Repair

Remove and dispose of damaged siding and trim.  Furnish and install new vinyl siding and new custom bent aluminum fascia as needed.  New siding and trim to match existing.  Installation to meet all current code requirements and proper technique to meet manufacturers written instructions.

Notes - Vinyl Siding to be a minimum thickness of 0.042 inches as manufactured by Wolverine "American Legend" or approved equal.  Location – Painted black siding only (rear entrance)

Aluminum trim to be a minimum thickness of 0.019 inches as manufactured by Wolverine-Petronix Inc., Alcoa, Alsco Anaconda Inc., or Reynolds. Location – Upper section only

46. NEW GUTTERS

$____________Spec:11.005   NEW GUTTERS

New Gutters

Remove and dispose of existing gutters completely. Furnish and install new 5-inch gutters, 3- inch downspouts, and 36-inch long extensions. Construct gutters and downspouts of 0.027-gauge seamless aluminum. Provide watertight construction fastened securely to structure. Color to be white.

Location: Entire house

86. CLEARANCE CLEANING

$____________Spec:26.2005   CLEARANCE CLEANING

Clearance Cleaning

After all work is complete, clean all areas of the house as required to eliminate lead dust and to obtain final clearance for lead based paint hazards. Clean using methods approved by HUD and the EPA.

Location: See Combination Lead Based Paint Inspection and Risk Assessment.

Note: Ingham County Land Bank will pay for two (2) lead clearance tests, interim and final. All additional or failed tests must be paid to the Ingham County Land Bank in advance of ordering additional tests.

87. PERMIT FOR PLUMBING CONTRACT

$____________Spec:26.2044   PERMIT FOR PLUMBING CONTRACT

Permit for Plumbing Contract

Cost of permit for Plumbing contract.

Note: Supply Ingham County Land bank with plumbing permit receipt when submitting for draw and finalized permit at the end of job.

92. REPLACE SANITARY PIPING AND TRAP

$____________Spec:16.055   REPLACE SANITARY PIPING AND TRAP

Replace Sanitary Piping and Trap

Replace sanitary piping with new sanitary piping and new sanitary trap including vent. Work to meet all current code requirements.

Location: At all necessary locations and as current code requires. (basement)

Note – Remove existing cast iron vent stack from second floor to exterior as needed.  Furnish and install new stack complete to exterior, as needed. Replace all damaged ceiling surfaces as needed.

94. FLOOR DRAIN

$____________Spec:02.001   GENERAL (TYPE IN ITEM TOPIC HERE)

Floor Drain

Furnish and install new floor drain, matching existing size.

Location: Basement, in existing location.

101. REPLACE LAUNDRY TUB

$____________Spec:16.072   REPLACE LAUNDRY TUB

Replace Laundry Tub

Replace existing laundry sink and faucet with a new laundry sink and faucet as manufactured by MUSTEE, UTILITUB 15F PREMIER structural thermoplastic laundry sink with adjustable legs. Sink to be 13 inches deep with a 19-gallon capacity and built-in scrub board. Provide a complete installation to include new faucet, drain, supply and sanitary piping and sanitary trap. Install per manufacturer’s instructions. Work to meet all current code requirements.

Location: Basement, in existing location.

106. PERMIT FOR MECHANICAL CONTRACT

$____________Spec:26.2045   PERMIT FOR MECHANICAL CONTRACT

Permit for Mechanical Contract

Cost for permit for Mechanical Contract.

Note: Supply Ingham County Land bank with mechanical permit receipt when submitting for draw and finalized mechanical permit at the end of job.

107.  95% EFFICIENT FURNACE

$____________Spec:17.011   95% EFFICIENT FURNACE

95% Efficient Furnace

Remove and dispose of furnace and related equipment completely. Provide a concrete base or approved equal to support new furnace a minimum of 2 inches above finished floor. Furnish and install a new gas fired Direct Vent high efficiency furnace, Trane or approved equal with a minimum efficiency of 95%. New furnace to be AGA Approved and have a minimum 20-year warranty on the heat exchanger. Balance system upon completion of installation. Connect new furnace to existing supply and return air plenums and ducts. Install new grills and registers. Contractors will be responsible for installing new ductwork for proper distribution to habitable rooms. Contractor to conceal new exposed ductwork when it cannot readily be concealed in existing surfaces. Furnish and install new Energy Star Compliant 5-2 Day programmable electronic thermostat as manufactured by Honeywell, model # TH4210D1005 or approved equal. Contractor will be responsible for proper venting of new furnace. Work to meet all current code requirements.

Location: Basement, in existing location.

Notes:

1)  Supply Ingham County Land Bank with documentation of furnace efficiency after installation.

2)  Furnish and install new, clean furnace filter at the end of job.

111. VENT FOR BATH FAN

$____________Spec:17.005   VENT FOR BATH FAN

Vent for Bath Fan

Furnish and install new rigid metal vent required to exhaust bathroom ventilation fan/light combination to daylight as required by current code. In unconditioned areas, provide insulation at vent pipe. Coordinate work with Electrical Contractor installing fan/light unit.

Location: Bathroom off kitchen

114. PERMIT FOR ELECTRICAL CONTRACT

$____________Spec:26.2046   PERMIT FOR ELECTRICAL CONTRACT

Permit for Electrical Contract

Cost of permit for electrical contract.

Note: Supply Ingham County Land Bank with electrical permit receipt when submitted for draw and finalized permit at the end of job.

117. CORRECT AND UPDATE WIRING ISSUES

$____________Spec:02.001   GENERAL (TYPE IN ITEM TOPIC HERE)

Correct and Update Wiring Issues

1)   Replace all damaged or unsightly switches, outlets and covers. Color to be white.

2)   Test and identify defective and ungrounded wiring at all switches, outlets and fixtures.

3)   Furnish and install new wiring where wire is deteriorated, unsafe per NEC.

4)   Furnish and install new GFCI outlets where required by code.

5)   Where needed, furnish and install new dedicated circuits for range, a/c, water heater, sump pump, dishwasher, refrigerator, garbage disposal and smoke detectors per NEC.

6)   Where needed, furnish and install dedicated circuits for kitchen.

7)   Install wiring and switches as needed for interior and exterior light fixtures.

8)  Furnish and install new LED bulbs in all fixtures.

9)  Furnish and install GFCI protected outlets at exterior front and rear of house.

10)  All new receptacles to be tamper proof.

11)  If sump pump is existing or to be added, furnish and install new sump pump outlet with 30mA interrupt rating in its own circuit.

12) Furnish and properly install new combo carbon monoxide detector/smoke detector 120 volt with battery back-up. Install according to manufacturer’s installation instructions and local code requirements.

121. BATH VENT AND LIGHT

$____________Spec:18.044   BATH VENT AND LIGHT

Bath Vent and Light

Furnish and install new exhaust fan with light/CFC bulb, as manufactured by NuTone, Model #769 RL, or approved equal, including new solid METAL ductwork, wall switch and GFCI branch circuit wiring per manufacturers installation instructions, to meet all current code requirements. Coordinate work with Mechanical Contractor installing vent piping.

Location: Bathroom

122. Recycling

$____________Spec:

Recycling

Recycling of all construction materials as needed.  Properly dispose of all left over unused construction materials from land-fill as needed.  As we moved towards a more circular environment, promote best practices in recycling and repurposing leftover construction material, by reducing the amount of materials to landfills.

Additional Notes:

At the conclusion of Construction:

All ducts are to be cleaned of any debris & all floor drains, sump crocks and clean-outs are to be cleaned of any debris

NOTE: Structural, Electrical, Plumbing and HAVC contractors to make all connections per Ingham County Land Bank selection sheets and specifications. All work to be done per local codes.

Contractor will be responsible to clean up the site as needed to have a safe environment during construction phases.

Bid Total $___________________________