Address:  (1727) Hillcrest, Lansing, MI 48910

Date:  5/23/23

4 Bedroom, 3 Bath Ranch with

Shed On Slab

General Notes, Product Specifications and Color Choices:

Energy Star Rating: Contractor shall use the following parameters to meet pre-determined Energy Star Plus Rating:

- Mechanical:   Provide and install a 95% efficient direct vent furnace to include all supply and return air ductwork and grills, including all gas lines for range and dryer.   Provide and install a new minimum 14 SEER Central Air Conditioner on pad, include all ventilation ductwork to meet code requirements. All ducts to be cleaned upon completion of all construction with new furnace filter.

- Water Heater:  Install a 50-gallon, gas fired, power vent water heater with a minimum 6-year warranty on tank and parts. Water heater must have first hour rating of 60; be Energy Star rated; have an energy factor of 0.82 or better; and must have a recovery rate of 79% or better.

- Wall Insulation:  Closed cellulose in all exterior wall cavities with a rating of 25, a maximum fuel contribution rating of 10, and a maximum smoke developed rating of 25. To have a thermal resistance value (R value) of 19.

- Attic Insulation: Install blown cellulose in attic area, achieve R-50. Install a barrier around eave vents and 2” x 4” framing at perimeter of attic access to keep insulation from blocking these opening. Install weather stripping and R-30 insulation to attic access panel. Installer shall provide certification of the thickness, settled thickness, and coverage area. Markers shall be provided for every 300 square feet of attic area, attached to trusses, rafters or joists and indicate in 1” high numbers the installed thickness of the insulation.

- Basement Wall Insulation: Furnish and install 1” rigid foam sheeting on exterior of basements walls. Parge wall to meet all code requirements. Provide and properly install 2 ½” Eco-Cell R-10 insulation on interior basement walls, hold 3 ½” off floor and install in metal track with weep holes.

- Sealing & caulking: new paintable silicone caulk at the bottom of the sill plate on the interior and exterior perimeter of house; all penetrations through exterior walls are to be completely sealed with caulk; caulk bottom plate on interior side; install approved tape to seal all heating duct and return air seams and boots prior to drywall finish; insulate and foam around all windows, doors, outlets, switches, and all other penetrations to the interior and exterior walls; prior to interior painting, install new paintable silicone caulk where all interior trim and drywall meet, i.e. window trim, door trim, baseboard trim. NOTE: All sealing and caulking to meet HERS rating guidelines and requirements.

- Windows: Minimum standard accepted is U value of 0.280, SHGC of 0.290

- Doors: Minimum standard accepted is U Value of 0.25, SHGC of 0.140

- Bond/Sill plates: Minimum standard of R-20 spray insulation

- Visitability Standard:

A no-step main floor entry through the front door with a threshold of less than 1/2“must be provided.

- All doorways on the main floor must be at least 32” wide with the door open at a 90-degree angle.  Note:  34” doors at the end of a hallway will generally require that the hallway be at least 40” wide to allow for door casings.

- All main floor bathrooms must include blocking for future grab bar installation which is not less than 33” and not more than 36” above the floor.   No grab bars being installed at this time.

- The main bathroom must have a clear floor space of 30” x 48” centered on & contiguous to the sink that is not encroached by the swing path of the bathroom door and a sink & toilet that allow for parallel or head-on approach by a person in a wheelchair.

- Main floor environmental controls, including light switches and heating/cooling controls must be located at least 15” and no more than 48” above the floor.

Site Plan:  Contractor provides final site plan and present for approval prior to construction beginning. A generalized Site Plan has been attached, but is NOT TO SCALE.   Contractor to include all site work required to provide house and site work described below.

Sitework/Drainage:  Fill and grade as required to blend with grade of lots in all directions.  Provide plan for drainage system from all downspouts and from sump discharge. Downspouts cannot discharge onto concrete drive or walks.

Landscaping:  Provide final grading of at least 6 inches of topsoil and seed; provide cover for seed.  Provide a 3-foot deep stone at the front of house; at edge of concrete walk from house to drive, including plastic edging, permanently installed and landscape fabric.  Plants to use should be Ornamental Grass, medium height across the front, with dwarf Alberta Spruce

flanking the house front; hostas in the front between the spruce. Stone to be 1”- 2¼” Michigan stone.

Foundation:  Contractor shall allow ICLB to inspect the foundation hole prior to installation of pea stone and concrete wall pour.

- Basement: Poured Concrete walls to be 10”, thick and 96” high on a 20” x 10” concrete footing. Include 1 egress window placed at rear of house per plan.

- Front Porch: Poured concrete wall to be 6” thick and 4’-0” high on a 12”x 6” concrete footing or as otherwise required due to existing soil conditions.

- Include passive radon mitigation system.  Note: all piping for this system above the basement ceiling shall be concealed in wall cavities.

- There must be a floor drain & related piping in the basement floor.  Provide and install supply lines for water and waste for new bathroom in basement.  Location:  See Foundation Plan

- Provide and install all framing and drywall required to finish interior and exterior of structure. NOTE: Basement bathroom and bedroom be included and to have a complete surface finish. Electrical and HAVC systems to be included in basement bathroom and bedroom.

NOTE: DO NOT drywall basement walls or ceiling beyond stairwell.  Contractor to provide and install a new handrail to basement as needed per local codes.

Concrete Flatwork:

Driveway:  Provide 10’ foot wide driveway from Public Street to new rear deck location (See site plan).  Install a new apron at the end of new drive way. Include all necessary permits.

Private Walk:  to meet Visitability Standards, include new private sidewalk (ramp) from driveway to front entry. Walkway to have proper slope to front porch slab.  Slope elevation to meet all requirements for serviceable to front door entrance. Walkway to be 60” wide to meet code requirements.

Deck:  Provide and install a new deck (14’x12’) and (2) two sets of steps at rear of house per approved site plan. Installation to be per local code requirements.

NOTE: All materials for rear deck to be wolmanized treated lumber materials secured with screws.

Plumbing:  Install PEX supply piping to all fixtures.  Provide chrome finished piping with escutcheons at all exposed piping areas.  Provide new sanitary piping to all fixtures to meet code, including sewage ejector where required.  Furnish and install laundry supply and sanitary piping per plan including laundry fittings. Provide and install new sill cocks at front and rear of unit. Provide and Install sump pump system complete.  (NOTE See Electrical requirements for the circuit servicing the sump).

- Toilets:  dual flush energy efficient, regular height American Standard or approved equal

- Tub & Surround Units:  Maax “Allia” Model # 6032, White or approved equal.

- Bathroom Showers:  Moen Posi-Temp single handle tub/shower trim kit with valve Model # T2133-BN-2570, Brushed Nickel finish or approved equal.

- Lavatory Faucets:   Moen lavatory faucet model #6410, Brushed Nickel finish or approved equal.

- Kitchen Sink:  Dayton Elite Double Bowl Sink Model DSE2333224, Stainless steel or approved equal.

- Kitchen Faucet:  Moen Camerist model # 7840, Classic Stainless finish or approved equal.

- Provide and install a new 32” white pedestal sink complete with hardware as needed. Randolph Morris or approved equal # RMC1315-4 (Main Bathroom)

- NOTE: Provide and install new vanity with top in the master and basement bathrooms. Vanities in master bedroom and basement bathroom to be 32”.

- Disposal:  Insinkerator food waste disposal model number CNTR333 with a minimum 3/4 hp motor, stainless steel flange, hopper and shredding mechanism and corrosion resistant housing or approved equal.

- Sump Pump & Waterproofing System – furnish and install new drain system to drain into a sump pump crock.  Furnish and install an automatic submersible sump pump, ½ HP, with approved check values.  Install discharge pipe to discharge well away from foundation, to grade.

Siding and trim:

- Siding Colors:  Body: CertainTeed MainStreet  Double 4” Clapboard in Cypress

- Gable Accent (House and Garage fronts):  CertainTeed Cedar Impressions Double 7" Straight Edge Rough-Split Shakes

- Color: Natural Clay

- Trim Color:  White

- 1” x 12” Azek, or approved equal, bond trim

- Color: White

- 4” House numbers – Black

- Aluminum Soffit – White

- Aluminum Fascia – White

- Vinyl Louver (gable) – Natural Clay

- Vinyl Shutters (front windows only) - Black

Gutters -Seamless

- Style:  5-inch

- Color: White

- Discharge to grade.

Shingles – CertainTeed

- Style:  Landmark Pro

Color:  Weathered Wood

Door Hardware (Exterior and interior)-Schlage

- Style: Grade 2 deadbolt, and Georgian handle style (Lever), keyed alike at front and side entries.

- Color:  Brushed Nickel- Lever handles

- Bed/bath lock at bedrooms and bathrooms, basement entry door-Lever handles

- Bi-Fold Closet Doors: Brushed Nickel Lever Handles

Door Hinges (Exterior and interior)

- Color:  Brushed Nickel

Interior Doors and Closet Doors:

Provide and install new 2-panel, square top, smooth solid core prehung doors, meeting visitability standards, and 2-panel, square top smooth bi-fold closet doors.

Closet rods and shelves

- Provide and install MDF bullnosed shelving with primed wood cleats. Provide new adjustable metal clothes rod with rod brackets

Exterior Doors: Therma- Tru Smooth Star

- Location:  Front & Rear

- Style: #S6021 1-panel half lite with integral blind. Lever handles

- Color:  Sherwin Williams SW  7006 Extra White – insulate header above

Locks:

- All doors to have Schlage Grade 2 deadbolts and Grade 3 cylindrical lock sets or approved equal, brushed nickel finish and match all doors- Lever handles

Storm Doors – Front & Rear

Style:  Anderson 3000 series - Full View or approval equal- Lever handle

Color:  White

Windows –

- Color:  White-exterior, White- Interior, White hardware

- Style:  Silverline by Andersen with Smart Sun Glass OR pre-approved equal

- Full snap screen; Double Hung new construction windows; Must meet Energy rating requirements stated on page 1;

Tilt Sashes; Air infiltration rate of 0.200 cfm/ft or less; multiple air chambers in window frame and sashes OR factory filled with foam; warm-edge spacer technology; overlapping and interlocking meeting rail; sloped or stepped sill on exterior; welded sash and frame; double lifetime limited warranty – insulate headers above

- Bathroom window to have frosted glass on lower sash and be tempered as required by code

Basement

- Provide and install new energy efficient glass block and vent assembly windows with minimum vent size of 16” x 8” with dual pane glass package and screens. Window wells to be included. See Foundation plans for location of all new cellar windows.

Interior Paint –Sherwin Williams, ProMar No VOC Paint - Interior:

Living Room, Dining Room, Kitchen, Hallway and Stairway

- Walls: Biscuit # SW 6112

Bedrooms and bathrooms walls:

- Window Pane # SW 6210

Trim/Doors throughout:  White

Ceilings: White

Kitchen Cabinets

- Style:  Aristokraft  Maple or approved equal

- Color:  Rouge

- Door Style:  Avalon

Bath Vanities Cabinets:

- Style:   Aristokraft  Maple or approved equal

- Color:  Rouge

- Door Style:  Avalon

NOTE:  Include 3 ¼” matching crown molding at all kitchen wall and tall cabinets, include toe kick and base shoe trim at all kitchen and bath base cabinets.

Cabinet Pulls

- Hardware: Element-Slape Collection

# 984-96 or approved equal

Kitchen Countertop

- Style:   Fine Velvet Finish

- Wilsonart # 4904-38 or approved equal

- Color:  Desert Springs

Bath countertops – US Marble

- Color:   Desert Sand, Model # 9189 or approved equal

Interior Trim:

- Empire 410 MDF colonial base trim 4 x 19/32

- Empire 411 MDF Colonial casing 3 x 11/16

NOTE all windows to have interior sills and aprons

Flooring

Kitchen, dining area, baths and laundry room

- Style:  Congoleum Air-Step Plus # 58040 or approved equal

- Color: Vanilla Cream

Living Room, Hallways, Bedrooms-Closets Carpet

- Style:  EcoVest PetMate Collection Pure-Color with SoilShields or approved equal

- Color: Driftwood

ELECTRICAL:

Minimum lighting standards: LED bulbs with minimum 60 W equivalent rating, a color temperature rating between 3000K-4000K. All halls, stairs and room necessary to cross to other rooms and stairways must be well lit and controlled by a 3-way switch using concealed wiring. Attics, basements & crawl spaces must have utility fixtures. All new light fixtures will be ENERGY STAR rated.  All fixtures chosen by contractor shall be of same or similar style. All kitchen receptacles to be GFI per local codes. All fixtures to be LED NOTE: New Basement Bathroom and Bedroom to be included.

Interior Lighting: LED

Kitchen Lighting (1):

- One mount or close ceiling fixture with minimum 2 bulbs Satin Nickel

- Model # 3074-11-65 Quorum or approved equal.

Dining Area:

- Three light chandelier style over table area, with brushed nickel finish

- Model # 621-3-65 Hemisphere or approved equal.

All Bedrooms (4), Walk In Closet (1), Hallway (1)

- Flush mount or close to ceiling mount, minimum 2 bulbs, Satin Nickel finish

- Model # 3066-11-65 Quorum or approved equal.

Entry (1) and Laundry Room (1):

- Flush mount or close to ceiling mount, minimum 2 bulbs, Satin Nickel finish

- Model # 3066-11-65 Quorum or approved equal.

Stairway

- Wall Sconce, satin nickel finish, with light illuminating steps

- Model # 5403-3-65 Quorum or approved equal.

Main Bathroom - Whole House Vents

Provide a new vent fan with light/LED bulb as manufactured by Nu-Tone, Model # ZN110ML, Ultrasense with motion Sensing Technology or approved equal, including new solid Metal ductwork and GFCI branch circuit wiring per manufacturers installation instructions, to meet all current code requirements. Coordinate work with Mechanical Contractor installing vent piping. Unit must have at least a 80 CFM boost function and be switched one of three ways: Switch at room entrance with adjustable time delay function that runs a period of time after the fan is turned off; or a motion detector with an adjustable time delay function that runs the fan for an additional period after the motion detector ceases to see motion; or by a humidistat. Indicate which method you are using with your bid.  Locations:  Main Bathroom, Master and basement

Master Bathroom above vanity mirror

- Model # 5108-3-64 Quorum or approved equal.

- Vanity Light with Satin Nickel finish, 3 bulbs

Bathroom above vanities mirror

- Model # 51-2- 64 Quorum or approved equal.

- Vanity Light with Satin Nickel finish, 2 bulbs

Exterior Light Fixtures:

Front Porch ceiling fixture:

- Model # 3086-11-6 Carrington-Quorum or approved equal.

- Outdoor Light, ceiling mount, White finish

- Ceiling fixtures for front porch to be Nora Lighting 4” LED 4000KNLLB-451D or approved equal.

Fixtures at rear deck, right side of door

- Model # 787-6 Cast Outdoor Designs or approved equal

- Wall Mount, White finish

Other:

- Sump Breaker to have 30mA Interrupting rating

- Basement lights should be on own circuit

- Plates, switches and outlets to be white.

- All locations to have combination smoke/CO detectors per code

Doorbell:

- Brushed Nickel finish

Appliances

Kitchen Appliances

- Elec.Range:  “Kenmore” Model # 73433, Stainless Steel or approved equal.

- Refrigerator:  “Kenmore Model # 51753 33” wide, Side by Side, Stainless Steel w/ice maker or approved equal.

- Dishwasher:  “Kenmore Model # 13803, Stainless Steel or approved equal.

- Microwave/Hood Combo:  “Kenmore” Model # 80333, Stainless Steel or approved equal.

- Microwave must be vented outside using heavy gauge galvanized metal or approved equal.

Laundry Appliances

- Washer: top load: “Kenmore” #26-22102000, white or approved equal.

- Dryer:  Elec, “Kenmore” #26-7120200, white or approved equal.

Home Hardware

Window Blinds

- Style: Bali Cut-to-Size or Blinds.com Economy Light Filtering Double Cell, Cellular Shade in white/cotton, cordless or approved equal

All Bathroom Hardware

- Style:  Classic

- Color:  Brushed Nickel

Hinge bumps/door bumps

- Style:  Door Bumps preferred; Hinge Bumps only if necessary

- Color:  Brushed Nickel

Shower rods (no tension)

- Style:  Curved

- Color:  Brushed Nickel

Mirrors (1/4” only)

- Size:  to match vanities

- Style: ¼” only frameless beveled

- Location: Over vanities

Mailbox

- Color:  White

- Style: Classic – with house numbers

- Location:  Mount on 4x4 post at street right-a-way or on front porch area of unit to meet USPS requirements

Radon Mitigation System:

Furnish and install a new active radon gas mitigation system complete. Furnish all required permits and facilitate all required inspections and approvals. Ensure proper combustion air to fuel burning appliances to prevent back draft from occurring. All work to be done by a National Environmental Professional, per ASTM standards, Designation E-2121-03. All Penetrations to be comply with applicable building, mechanical, plumbing, and electrical, energy and fire prevention codes and standards and regulations of the City Of Lansing. Supply Ingham County Land Bank with a copy of permit and final approval documentations.

Recycling of all construction materials:

As we move towards a more circular environment, promote best practices in recycling and repurposing leftover construction material, by reducing the amount of material to landfills. This includes all trades. Structural, Electrical, Plumbing and HAVC

SHED WITH PAD: Excavate to provide sufficient depth to install a 4-inch thick concrete pad on 4-inch minimum compacted sand fill for new shed 8’ feet wide by 13” feet deep. Provide adequate slope to drain water away from the structure. Furnish and install a new gambrel style wood shed, 8” foot by 13’ foot, including new framing, 7/16 inch OSB wall and roof sheathing 3/4” inch OSB floor sheathing (if shed trusses are used). Include new vinyl siding and trim to match new house siding and trim. Include new roofing materials and asphalt shingles to match new house roofing. Include a double 24” inch entry reinforced doors with pulls, stops hasp, and keyed padlock. Provide Ingham County Land Bank with keys immediately upon installation of padlock. All work to be done to ensure proper structural integrity and meet all current codes and zoning requirement. Location: South-West corner section of property.

Additional Notes:

At the conclusion of Construction:

 All ducts are to be cleaned of any debris & all floor drains, sump crocks and clean-outs are to be cleaned of any debris

 Sewer line is to be cleaned out and video’ed from basement cleanout to the street connection and copy of video to be provided to ICLB upon completion.

BASEMENT BATHROOM AND BEDROOM TO BE INCLLUDED WITH ALL: WALLS/CEILINGS/DOORS AND FLOORS ITEMS IN THIS STRUCTURAL SPECIFICATION SELECTION SHEET