Address:  (1108) E Cesar E Chavez, Lansing 48906

Date:  5/23/23

3 Bedroom, 2 Bath Two Story with

Detached Garage

General Notes, Product Specifications and Color Choices:

Energy Star Rating: Contractor shall use the following parameters to meet pre-determined Energy Star Plus Rating:

- Mechanical:   Provide and install a 95% efficient direct vent furnace to include all supply and return air ductwork and grills, including all gas lines for range and dryer.   Provide and install a new minimum 14 SEER Central Air Conditioner Include all ventilation ductwork to meet code requirements. All ducts to be cleaned upon completion of all construction. NOTE: HAVC contractor to make all connections and seal all seams on ductwork as needed.

- Water Heater:  Install a 50-gallon, gas fired, power vent water heater with a minimum 6-year warranty on tank and parts. Water heater must have first hour rating of 60; be Energy Star rated; have an energy factor of 0.82 or better; and must have a recovery rate of 79% or better.

- Wall Insulation: Closed cellulose in all exterior wall cavities with a rating of 25, a maximum fuel contribution rating of 10, and a maximum smoke developed rating of 25. To have a thermal resistance value (R value) of 19.

- Attic Insulation: Install blown cellulose in attic area, achieve R-50. Install a barrier around eave vents and 2”x4” framing at perimeter of attic access to keep insulation from blocking these opening. Install weather stripping and R-30 insulation to attic access panel. Installer shall provide certification of the thickness, settled thickness, and coverage area. Markers shall be provided for every 300 square feet of attic area, attached to trusses, rafters or joists and indicate in 1” high numbers the installed thickness of the insulation.

- Basement Wall Insulation: Furnish and install 1” rigid foam sheeting on the exterior of the basement walls. Parge wall to meet all code requirements. Provide and properly install 2 ½ “Eco-cell R-10 insulation on the interior basement walls, hold 3 ½ “off floor and install in metal track with weep holes.

- Sealing & Caulking: New paintable silicone caulk at the bottom of the sill plate on the interior and exterior perimeter of house; all penetrations through exterior walls are to be completely sealed with caulk; caulk bottom plate on the interior side; install approved tape to seal all heating duct and return air seams and boots prior to drywall finish; insulate and foam around all windows, doors ,outlets, switches, and all other penetrations to the interior and exterior walls; prior to interior painting, install new paintable silicone caulk where all interior trim and drywall meet, i.e. window trim, door trim, baseboard trim. NOTE: All sealing and caulking to meet HERS rating guidelines and requirements. NOTE: garage to be included.

- Windows: Minimum standard accepted is U value of 0.280, SHGC of 0.290

- Doors: Minimum standard accepted is U Value of 0.25, SHGC of 0.140

- Patio Door: Minimum standard accepted is U Value of 0.31, SHGC of .0170 ( if needed)

- Bond/Sill plates: Minimum standard of R-20 spray insulation

Visitability Standard:

- A no-step main floor entry through the front door with a threshold of less than ½” must be provided.

- All doorways on the main floor must be at least 32” wide with the door open at a 90-degree angle. Note: 34” doors at the end of a hallway will generally require that the hallway be at least 40” wide to allow for door casings.

- All main floor bathrooms must include blocking for future grab bar installation which is not less than 33” and not more than 36” above the floor.

- The main bathroom must have a clear floor space of 30”x 48” centered on & contiguous to the sink that is not encroached by the swing path of the bathroom door and a sink & toilet that allow for parallel or head-on approach by a person in a wheelchair.

- Main floor environmental controls, including light switches and heating/cooling controls must be located at least 15” and no more than 48” above the floor.

Site Plan:  Contractor provides final site plan and present for approval prior to construction beginning. A generalized Site Plan has been attached, but is NOT TO SCALE.   Contractor to include all site work required to provide house and site work described below.

Site work / Drainage:  Fill and grade as required to blend with grade of lots in all directions.  Provide plan for drainage system from all downspouts and from sump discharge. Downspouts cannot discharge onto concrete drive or walks.

Tree and Brush Removal:   Remove all trees, stumps and brush from site (will be marked with paint dots). Rake lot and remove all downed branches and brush. Remove all brushy vegetation growing at rear yard areas. Removal of trees and brushy vegetation.

Landscaping:  Provide final grading of at least 6 inches of topsoil and seed, provide cover for seed.  Provide a 3-foot deep stone at the front of house; at edge of concrete walk from house to drive; and is to include plastic edging, permanently installed and landscape fabric.  Plants to use should be Ornamental Grass – medium height across the front, with dwarf Alberta Spruce flanking the house front; hostas in the front between the spruce. Stone to be 1’ -2 ¼ “Michigan stone.

Foundation:  Contractor shall allow ICLB to inspect the foundation hole prior to installation of pea stone and concrete wall pour.

- Basement: Poured Concrete walls to be 8” thick and 96” high on a 16” x 8” concrete footing.  Include 1 egress windows placed at rear of house. (South/West Elevation)

- Front Porch: Poured concrete wall to be 6” thick and 48” high on a 12”x 6” concrete footing or as otherwise required due to existing soil conditions.

- Include passive radon mitigation system. Note: all piping for this system above the basement ceiling shall be concealed in wall cavities.

- There must be a floor drain & related piping in the basement floor.  Provide and stub-in supply lines for future bathroom in basement. Location: See Foundation Plan NOTE: Plumbing contractor to properly install all water, sewer and waste lines complete as needed per code.

- Provide and install all framing and drywall required to finish interior and exterior of structure.

NOTE:  Do not drywall basement walls or ceiling beyond stairwell.  Contractor to provide and install a new handrail to basement as needed per local codes.

Concrete Flatwork:

Driveway: Provide and install a 12’ foot wide to the length of the detached garage slab. Install a new apron at the end of new drive way. Include all necessary permits. NOTE: Garage slab to be included.

Private Walk: Include new private sidewalk (ramp) from driveway to front entry. Walk way to have proper slope to front porch. Slope elevation to meet all requirements for visitability, assessible to front door entrance. Walk way to be 36 inches wide to meet code requirements.  See plot plan for dimensions and layout to front porch area.

Deck:  Provide and install a new (14’x12’) rear wolamizied deck complete with steps, guardrails, handrails, and spindles complete, off kitchen area of home. See prints for construction details.

Plumbing:  Install PEX and PVC supply piping to all fixtures.  Provide chrome finished piping with escutcheons at all exposed piping areas.  Provide new sanitary piping to all fixtures to meet code, including sewage ejector where required.  Furnish and install laundry supply and sanitary piping per code including laundry fittings on 2nd floor area. Note: Plumbing contractor is responsible for all connections from the unit to the main street connections, water and sewer including all connections to the new unit as needed to meet all local codes. Sill cocks to be included front and rear of unit.

NOTE:  See electrical requirements for the circuit servicing the sump.

- Toilets: Dual flush energy efficient, regular height. American Standard or approved equal

- Tub & Surround: Maax “Allia” model # 6032, White or approved equal

- Bathroom Shower: Moen Posi-Temp single handle tub/shower trim kit with valve Model # T2133-BN-2570, Brushed Nickel finish or approved equal

- Lavatory Faucets: Moen lavatory faucet model # 6410 Brushed Nickel finish or approved equal

- Kitchen Sink: Dayton Elite Double Bowl Sink Model # DSE2333224, Stainless steel or approved equal

- Kitchen Faucet: Moen Camerist model # 7840, Classic Stainless finish or approved equal

- Provide and install a new 32” White Pedestal sink complete with hardware as needed. Randolph Morris or approved equal (Bathroom off kitchen)

- Disposal: Insinkerator food waste disposal model # CNTR333 with a minimum ¾ H, stainless steel flange, hopper and shredding mechanism and corrosion

- Sump Pump & Waterproofing System- Furnish and install new drain system to drain into a sump pump crock. Furnish and install an automatic submersible sump pump ½ HP with approved check values. Install discharge pipe to discharge well away from foundation, to grade.

Siding and trim:  House and Garage

- Vinyl Shutters-Black - Front Windows Only

- 4” House numbers- White (House)

- Siding Colors: Body: CertainTeed MainStreet Double 4” Clapboard in Cypress

- Gable Accent (House and Garage): CertainTeed Cedar Impressions Double 7” Straight Edge Rough-Split Shakes

- Color: Natural Clay

- Trim Color: White

- Aluminum Soffit- White

- Aluminum Fascia- White

Gutters – Seamless (House & Garage)

- Style: 5 inch

- Color: White

- Discharge to grade away from foundation system

Shingles - CertainTeed- House & Garage

- Style: Landmark Pro

- Color: Weathered Wood

Storm Doors – Front & Rear

- Style:  Anderson 3000 series - Full View or approval equal -Lever handle

- Color:  White

- Basement Windows: See Foundation Plan

Garage Overhead Door:  Clopay, ” Value Series”:

- Garage Door Opener:  Liftmaster 3265

- Manufactured:  Chamberlain with key pad

- Color:  White (Garage Door)

Exterior Doors:  Therma Tru, “Profiles “Fiberglass/Steel

- Style: Half Circle Light,

- Locations: House and garage

- Color: White

Locks:  All doors to have Schlage Grade 2 deadbolts and Grade 3 cylindrical lock sets or approved equal, brushed nickel finish and match all doors-Lever handles

Door Hinges (exterior and interior):

Brushed Nickel

Door Knobs (Exterior and Interior) -Schlage Style:  Grade 2 deadbolt, and Georgian Knob, keyed alike at front & rear entries Lever handles

Interior Doors and Closet Doors:

Provide and install new 2-panel, square top, smooth solid core perhung doors meeting visitability standards, and 2-panel, square top smooth bi- fold doors

Closet rods and shelves:  Provide and install MDF bullnosed shelving with primed wood cleats. Provide new adjustable metal clothes rod with rod brackets

Windows:

Color: White

Style: Silverline by Andersen with smart sun glass or approved equal. Full snap screen; Double hung new construction windows; Must meet Energy rating requirements stated on page 1: Tilt Sashes; Air infiltration rate of 0.200 cfm/ft or less; multiple air chambers in window frame and sashes OR factory filled with foam; warm-edge spacers technology; overlapping and interlocking meeting rail; sloped or stepped sill on exterior sash and frame; double lifetime limited warranty-insulated header above.

Bathroom window: to have frosted glass on lower sash and be tempered as required by

code. NOTE Garage to have (2) two windows one on each side.

Interior Paint – Sherwin Williams, ProMar No VOC Paint

Interior:  Living Room, Dining Room, Kitchen, Hallway and Stairway

- Walls: Biscuit # SW 6112

- Bedrooms and bathrooms:  Window Pane     # SW 6210

- Trim/Doors throughout:  White

- Ceilings: White

- Kitchen Cabinets:

- Style: Arisokraft Maple or approved equal

- Color: Rouge

- Door Style: Avalon

Bathroom Vanity Cabinets:

Style: Aristokraft Maple or approved equal

Color: Rouge

Door Style: Avalon

NOTE: Include 3 ¼ matching crown molding all kitchen wall and tall cabinets, include toe kick and base shoe trim at all kitchen and bath base cabinets.

Cabinet Pulls:

Hardware: Element-Slape Collection # 984-96 or approved equal

Kitchen Countertop:

Style: Fine Velvet Finish

Wilsonart # 4904-38 or approved equal

Color: Desert Springs

Bathroom Countertop:

Color: Desert Sand, Model # 9189 or approved equal

Interior Trim:

Empire 410 MDF colonial base trim 4x19/32

Empire 411 MDF colonial casing 3x11/16

NOTE: all windows to have interior sills and aprons

Flooring

Kitchen, dining area, bathrooms

- Style:  Congoleum Air-Step Plus # 58040 or approval equal

- Color:  Vanilla Cream

- NOTE: Front entrance Living Room area to have a 4’ x4’ area at doorway installed

Living Room, 2nd Floor Hallway, bedrooms and Stairway to 2nd Floor

- Style:  EcoVest PetMate Collection PureColor with SoilShields or approved equal

- Color:  Driftwood

ELECTRICAL:

Minimum lighting standards: LED bulbs with minimum 60 W equivalent rating, a color temperature rating between 3000K-4000K. All halls, stairs and room necessary to cross to other rooms and stairways must be well lit and controlled by a 3-way switch using concealed wiring. Attics, basements & crawl spaces must have utility fixtures. All new light fixtures will be ENERGY STAR rated.  All fixtures chosen by contractor shall be of same or similar style

NOTE: Electrical contractor is responsible for all connections from the unit to the exterior service pole and all interior connections.

NOTE: Electrical contractor to install new control switches at top and bottom of new stairway to basement as need per local codes. Contractor to properly install new GFI plugs in kitchen as per local codes. All Fixtures to be LED

Interior Lighting:

Kitchen Lighting (1):

- One mount or close ceiling fixture with minimum 2 bulbs Satin Nickel

- Model # 3074-11-65 Quorum

Dining Area:

- Three light chandelier style over table area, with brushed nickel finish

- Model # 621-3-65 Hemisphere or approved equal

All Bedrooms (3), Walk-In Closet (1) Hallway (1)

- Flush mount or close to ceiling mount, minimum 2 bulbs, Satin Nickel finish

- Model # 3066-11-65 Quorum or approved equal

Entry (1):

- Flush mount or close to ceiling mount, minimum 2 bulbs, Satin Nickel finish

- Model # 3066-11-65 Quorum

Stairway

- Wall Sconce, satin nickel finish, with light illuminating steps

- Model # 5403-3-65 Quorum or approved equal

Main Bathroom - Whole House Vent

- Provide a new vent fan with light/LED bulb, as manufactured by Nu-Tone Model # ZN100ML, Ultrasense with Motion Sensing Technology or approved equal, including new solid Metal ductwork and GFIC branch circuit wiring per manufacturers installation instructions, to meet all current code requirements. Coordinate work with Mechanical contractor installing vent piping. Unit must have at least a 80 CFM boost function and be switched one of three ways: Switch at room entrance with adjustable time delay function that runs a period of time after fan is turned off; or a motion detector with an adjustable time delay function that runs the fan for an additional period after the motion detector ceases to see motion; OR by a humidistat. Indicate which method you are using with your bid.   Location: Main Bathroom

Master Bathroom above vanity mirror

- Vanity Light with Satin Nickel finish, 2 bulbs

- Model # 5108-3-64 Quorum or approved equal

Bathroom above vanity mirror

- Vanity Light with Satin Nickel finish, 2 bulbs

- Model # 5108-2-64 Quorum or approved equal

Exterior Light Fixtures:

Front Porch ceiling fixture:

Outdoor Light, ceiling mount, White finish

Wall fixture at rear deck right side:

Wall Mount, White finish, Model # 787-6 Cast Outdoor Designs or approval equal

NOTE: Ceiling fixtures for front porch to be Nora lighting 4” LED 4000KNLLB-451D or approved equal

Fixture at rear dreck:

Model # 787-6 Cast Outdoor Designs or approved equal.

Wall Mount, White finish

Other:

- Basement lights should be on own circuit

- Plates, switches and outlets to be white.

- All locations to have combination smoke/CO detectors

Doorbell:

- Brushed Nickel finish

Appliances

Kitchen Appliances

- Elec.Range:  “GE”  Model # JB64RKSS Stainless Steel or approval equal

- Refrigerator:  “Kenmore” Model # 51753 33” wide, Side by Side, Stainless Steel w/ice maker or approved equal

- Dishwasher:  “Kenmore” Model #13803, Stainless Steel or approved equal

- Microwave/Hood Combo:  “Kenmore”  Model # 80333, Stainless Steel or approved equal

- Microwave must be vented outside using heavy gauge galvanized metal

Laundry Appliances

- Washer: top load: “Kenmore” #26-22102000, white or approved equal

- Dryer:  Gas, “Kenmore” #26-7120200, white or approved equal

Home Hardware

Window Blinds

- Style: Bali Cut-to-Size or Blinds.com Economy Light Filtering Double Cell, Cellular Shade in white/cotton, cordless

Shower rods (no tension)

- Style:  Cured

- Color:  Brushed Nickel

Mirrors (1/4” only)

- Size:  to match vanity

- Style: ¼” only frameless beveled

- Location:  Over vanities

- All Bathroom Hardware:

Style: Classic

Color: Brushed Nickel

Hinge bumps/Door bumps:

Style: Door Bumps preferred;; Hinge Bumps

Only if necessary

Color: Brushed Nickle

Mailbox

- Style: Classic

- Color:  White  with house numbers

- Location: Mounted at front  porch exterior wall to meet USPS requirements or install 4”x 4”post with mailbox and number at street right away

Radon Mitigation System:

Furnish and install a new active radon gas mitigation system complete. Furnish all required permits and facilitate all required inspections and approvals. Ensure proper combustion air to fuel burning appliances to prevent back draft from occurring. All work to be done by a National Environmental Professional, per ASTM standards, Designation E-2121-03. All Penetrations to be comply with applicable building, mechanical, plumbing, and electrical, energy and fire prevention codes and standards and regulations of the City Of Lansing. Supply Ingham County Land Bank with a copy of permit and final approval documentations.

Recycling of all construction materials:

As we move towards a more circular environment, promote best practices in recycling and repurposing leftover construction material, by reducing the amount of material to landfills. This includes all trades. Structural, Electrical, Plumbing and HAVC

Additional Notes:

At the conclusion of Construction:

 All ducts are to be cleaned of any debris & all floor drains, sump crocks and clean-outs are to be cleaned of any debris

 Sewer line is to be cleaned out and video’ed from basement cleanout to the street connection and copy of video to be provided to ICLB upon completion.

NOTE: Structural, Electrical, Plumbing and HAVC contractors to make all connections per Ingham County Land Bank selection sheets and specifications sheet. All work to be done per local codes.