Address:  (910) N Chestnut Street Lansing 48906

Date:  5/23/23

4 Bedroom, 2.5 Bath Two Story with

Shed on a slab

General Notes, Product Specifications and Color Choices:

Energy Star Rating: Contractor shall use the following parameters to meet pre-determined Energy Star Plus Rating:

- Mechanical:   Provide and install a 95% efficient direct vent furnace to include all supply and return air ductwork and grills, including all gas lines for range and dryer.   Provide and install a new minimum 14 SEER Central Air Conditioner Include all ventilation ductwork to meet code requirements. All ducts to be cleaned upon completion of all construction. NOTE: HAVC contractor to make all connections and seal all seams on ductwork as needed.

- Water Heater:  Install a 50-gallon, gas fired, power vent water heater with a minimum 6 year warranty on tank and parts. Water heater must have first hour rating of 60; be Energy Star rated; have an energy factor of 0.82 or better; and must have a recovery rate of 79% or better.

- Wall Insulation: Closed cellulose in all exterior wall cavities with a rating of 25, a maximum fuel contribution rating of 10, and a maximum smoke developed rating of 25. To have a thermal resistance value (R value) of 19.

- Attic Insulation: Install blown cellulose in attic area, achieve R-50. Install a barrier around eave vents and 2”x4” framing at perimeter of attic access to keep insulation from blocking these opening. Install weather stripping and R-30 insulation to attic access panel. Installer shall provide certification of the thickness, settled thickness, and coverage area. Markers shall be provided for every 300 square feet of attic area, attached to trusses, rafters or joists and indicate in 1” high numbers the installed thickness of the insulation.

- Basement Wall Insulation: Furnish and install 1” rigid foam sheeting on the exterior of the basement walls. Parge wall to meet all code requirements. Provide and properly install 2 ½ “Eco-cell R-10 insulation on the interior basement walls, hold 3 ½ “off floor and install in metal track with weep holes.

- Sealing & Caulking: New paintable silicone caulk at the bottom of the sill plate on the interior and exterior perimeter of house; all penetrations through exterior walls are to be completely sealed with caulk; caulk bottom plate on the interior side; install approved tape to seal all heating duct and return air seams and boots prior to drywall finish; insulate and foam around all windows, doors ,outlets, switches, and all other penetrations to the interior and exterior walls; prior to interior painting, install new paintable silicone caulk where all interior trim and drywall meet, i.e. window trim, door trim, baseboard trim. NOTE: New basement bathroom and bedroom to be included. All sealing and caulking to meet HERS rating guidelines and requirements.

- Windows: Minimum standard accepted is U value of 0.280, SHGC of 0.290

- Doors: Minimum standard accepted is U Value of 0.25, SHGC of 0.140

- Patio Door: Minimum standard accepted is U Value of 0.31, SHGC of .0170 (if needed)

- Bond/Sill plates: Minimum standard of R-20 spray insulation

Visitability Standard:

- A no-step main floor entry through the front door with a threshold of less than ½” must be provided.

- All doorways on the main floor must be at least 32” wide with the door open at a 90 degree angle. Note: 34” doors at the end of a hallway will generally require that the hallway be at least 40” wide to allow for door casings.

- All main floor bathrooms must include blocking for future grab bar installation which is not less than 33” and not more than 36” above the floor.

- The main bathroom must have a clear floor space of 30”x 48” centered on & contiguous to the sink that is not encroached by the swing path of the bathroom door and a sink & toilet that allow for parallel or head-on approach by a person in a wheelchair.

- Main floor environmental controls, including light switches and heating/cooling controls must be located at least 15” and no more than 48” above the floor.

Site Plan:  Contractor provides final site plan and present for approval prior to construction beginning. A generalized Site Plan has been attached, but is NOT TO SCALE.   Contractor to include all site work required to provide house and site work described below.

Site work / Drainage:  Fill and grade as required to blend with grade of lots in all directions.  Provide plan for drainage system from all downspouts and from sump discharge. Downspouts cannot discharge onto concrete drive or walks.

Tree and Brush Removal:   Remove all trees, stumps and brush from site (will be marked with paint dots). Rake lot and remove all downed branches and brush. Remove all brushy vegetation growing at rear yard areas. Removal of trees and brushy vegetation.

Landscaping:  Provide final grading of at least 6 inches of topsoil and seed, provide cover for seed.  Provide a 3-foot deep stone at the front of house; at edge of concrete walk from house

to drive; and is to include plastic edging, permanently installed and landscape fabric.  Plants to use should be Ornamental Grass – medium height across the front, with dwarf Alberta Spruce flanking the house front; hostas in the front between the spruce. Stone to be 1’ -2 ¼ “Michigan stone.

Foundation:  Contractor shall allow ICLB to inspect the foundation hole prior to installation of pea stone and concrete wall pour.

- Basement: Poured Concrete walls to be 8” thick and 96” high on a 16” x 8” concrete footing.  Include 1 egress windows placed at rear of house. (South/West Elevation)

- Front Porch: Poured concrete wall to be 6” thick and 48” high on a 12”x 6” concrete footing or as otherwise required due to existing soil conditions.

- Include passive radon mitigation system.  Note: all piping for this system above the basement ceiling shall be concealed in wall cavities.

- There must be a floor drain & related piping in the basement floor.  Provide and install new supply lines for bathroom in basement complete. Location: See Foundation Plan NOTE: Plumbing contractor to properly install all water, sewer and waste lines complete as needed per code.

- Provide and install all framing and drywall required to finish interior and exterior of structure.

NOTE: Do not drywall basement walls or ceiling beyond stairwell. NOTE: Contractor to finish new bedroom and bathroom in basement complete as per (Walls/ceiling/floor and doors). All Electrical and HAVC to be included Contractor to provide and install a new handrail to basement as needed per local codes.

Concrete Flatwork:

Driveway: Provide and install a 12’ foot wide to the length of new rear deck area. Install a new apron at the end of new drive way. Include all necessary permits.

Private Walk: Include new private sidewalk (ramp) from driveway to front entry. Walk way to have proper slope to front porch. Slope elevation to meet all requirements for visitability, assessible to front door entrance. Walk way to be 36 inches wide to meet code requirements.  See plot plan for dimensions and layout to front porch area.

Deck:  Provide and install a new (14’x12’) wolmanized rear deck complete off kitchen area of home with (2) sets of steps, guardrails and intermediated as per approved site plan. Installation to be per local code requirements. NOTE: All decking and steps to be secured with screws.

Door Hardware (Exterior and Interior)-Schlage

Style: Grade 2 deadbolt, and Georgian handle style (Lever), keyed alike at front and rear entries.

Color: Brushed Nickel- Lever handles

Bedrooms & Bathrooms locks to be lever handles

Bi-fold Closet Doors: Brushed Nickel lever handles

Door Hinges: (Exterior & Interior)

Color: Brushed Nickel

Interior Doors and Closet Doors:

Provide and install new 2-panel, square top, smooth solid core perhung doors, meeting vistibility standards, and 2-panel, square top smooth bi-fold closet doors.

Closet rods and shelves

Provide and install MDF bullnosed shelving with primed wood cleats. Provide new adjustable metal clothes rod with rod brackets

Exterior Doors: Therma-Tru Smooth Star

Location: Front and Rear

Style: # S6021 1-panel half lite with integral blind. Lever handles

Color: Sherwin Williams SW- 7006 Extra White- insulate header above

Storm Doors-Front & Rear:

Style: Anderson 3000 series- Full view or approval equal-Lever handle

Color: White

Windows:

Color: White-exterior, White-Interior, White hardware

Style: Style: Silverline by Anderson with Smart Sun Glass OR pre-approved equal

Full snap screen; Double Hung new construction windows; Must meet Energy rating requirements stated on page 1: Tilt Sashes: Air infiltration rate of 0.200 cfm/ft. or less; multiple air chambers in window frame and sashes OR factory filled with foam; warm-edge spacer technology; overlapping and interlocking meeting rail; sloped or stepped sill on exterior; welded sash and frame; double lifetime limited warranty-insulate headers above.

Bathroom Window: To have frosted glass on lower sash and be tempered as required by code

Plumbing:  Install PEX and PVC supply piping to all fixtures.  Provide chrome finished piping with escutcheons at all exposed piping areas.  Provide new sanitary piping to all fixtures to meet code, including sewage ejector where required. Sill cocks to be included in front and rear of unit. Furnish and install laundry supply and sanitary piping per code including laundry fittings in basement. Provide and install new sill cocks to the front and rear of unit complete. Note: Plumbing contractor is responsible for all connections from the unit to the main street connections, water and sewer including all connections to the new unit as needed to meet all local codes.

NOTE: See electrical requirements for the circuit servicing the sump.

- Toilets: Dual flush energy efficient, regular height. American Standard or approved equal

- Tub & Surrounds: Maax “Allia” model # 6032, White or approved equal

- Bathroom Showers: Posi-Temp single handle tub/shower trim kit with valve Model # T2133-BN-2570, Brushed Nickel finish or approved equal

- Lavatory Faucets: Moen lavatory faucet model # 6410 Brushed Nickel finish or approved equal

- Kitchen Sink: Dayton Elite Double Bowl Sink Model # DSE2333224, Stainless steel or approved equal

- Kitchen Faucet: Moen Camerist model # 7840, Classic Stainless finish or approved equal

- Provide and install a new 32” White Pedestal sink complete with hardware as needed. Randolph Morris or approved equal (Half Bath Off Kitchen)

- Disposal: Insinkerator food waste disposal model # CNTR333 with a minimum ¾ H, stainless steel flange, hopper and shredding mechanism and corrosion or approved equal

- Sump Pump & Waterproofing System- Furnish and install new drain system to drain into a sump pump crock. Furnish and install an automatic submersible sump pump ½ HP with approved check

values. Install discharge pipe to discharge well away from foundation, to grade.

Siding and trim:

- Vinyl Shutters-Black - Front Windows Only

- 4” House numbers- White

- Siding Colors Body :CertainTeed MainStreet Double 4” Clapboard in Cypress

- Gable Accent (House Front) CertainTeed Cedar Impressions Double 7” Straight Edge Rough-Split Shakes

- Color: Natural Clay

- Trim Color: White

- Aluminum Soffit- White

- Aluminum Fascia- White

Gutters -Seamless

- Style: 5 inch

- Color: White

- Discharge to grade away from foundation system

Shingles-CertainTeed:

- Style: Landmark Pro

- Weathered Wood

Storm Doors – Front & Rear

- Style:  Anderson 3000 series - Full View or approval equal -Lever handle

- Color:  White

- Basement Windows: See Foundation Plan

Interior Paint – Sherwin Williams, ProMar No VOC Paint

Interior:  Living Room, Dining Room, Kitchen, Hallway and Stairway

- Walls: Biscuit # SW 6112

- Bedrooms and bathrooms:  Window Pane     # SW 6210

- Trim/Doors throughout:  White

- Ceilings: White

Flooring

Kitchen, dining area, bathrooms and laundry room

- Style:  Congoleum Air-Step Plus # 58040 or approval equal

- Color:  Vanilla Cream

NOTE: Front entrance Living Room area to have a 4’ x4’ area at doorway installed

Living Room, 2nd Floor Hallway, bedrooms and Stairway to 2nd Floor

- Style:  EcoVest PetMate Collection PureColor with SoilShields or approved equal

- Color:  Driftwood

ELECTRICAL:

Minimum lighting standards: LED bulbs with minimum 60 W equivalent rating, a color temperature rating between 3000K-4000K. All halls, stairs and room necessary to cross to other rooms and stairways must be well lit and controlled by a 3-way switch using concealed wiring. Attics, basements & crawl spaces must have utility fixtures. All new light fixtures will be ENERGY STAR rated.  All fixtures chosen by contractor shall be of same or similar style

NOTE: Electrical contractor is responsible for all connections from the unit to the exterior service pole and all interior connections after the unit has been placed on the new foundation system. NOTE: Electrical contractor to install new control switches at top and bottom of new stairway to basement as need per local codes. GFI to be installed in kitchen as per local codes. All fixtures to be LED.

Interior Lighting:

Kitchen Lighting (1):

- One mount or close ceiling fixture with minimum 2 bulbs Satin Nickel

- Model # 3074-11-65 Quorum or approved equal

Dining Area:

- Three light chandelier style over table area, with brushed nickel finish

- Model # 621-3-65 Hemisphere or approved equal

All Bedrooms (4), Walk In Closet (1) Hallway (1)

- Flush mount or close to ceiling mount, minimum 2 bulbs, Satin Nickel finish

- Model # 3066-11-65 Quorum or approved equal

Entry (1):

- Flush mount or close to ceiling mount, minimum 2 bulbs, Satin Nickel finish

- Model # 3066-11-65 Quorum or approved equal

Stairway

- Wall Sconce, satin nickel finish, with light illuminating steps

- Model # 5403-3-65 Quorum or approved equal

Main Bathrooms - Whole House Vent

- Provide a new vent fan with light/LED bulb, as manufactured by Nu-Tone Model # ZN100ML, Ultrasense with Motion Sensing Technology or approved equal, including new solid Metal ductwork and GFIC branch circuit wiring per manufacturers installation instructions, to meet all current code requirements. Coordinate work with Mechanical contractor installing vent piping. Unit must have at least a 80 CFM boost function and be switched one of three ways: Switch at room entrance with adjustable time delay function that runs a period of time after fan is turned off; or a motion detector with an adjustable time delay function that runs the fan for an additional period after the motion detector ceases to see motion; OR by a humidistat. Indicate which method you are using with your bid.   Location: Main Bathrooms

Master Bathroom above vanity mirror

- Vanity Light with Satin Nickel finish, 2 bulbs

- Model # 5108-3-64 Quorum or approved equal

Bathroom above vanity mirror

- Vanity Light with Satin Nickel finish, 2 bulbs

- Model # 5108-2-64 Quorum or approved equal

Exterior Light Fixtures:

Front Porch ceiling fixture:

Outdoor Light, ceiling mount, White finish

Wall fixture at rear deck right side:

Wall Mount, White finish, Model # 787-6 Cast Outdoor Designs or approval equal

Other:

- Basement lights should be on own circuit

- Plates, switches and outlets to be white.

- All locations to have combination smoke/CO detectors

Doorbell:

- Brushed Nickel finish

Appliances

Kitchen Appliances

- Elec.Range:  “GE”  Model # JB64RKSS Stainless Steel or approval equal

- Refrigerator:  “Kenmore” Model # 51753 33” wide, Side by Side, Stainless Steel w/ice maker or approved equal

- Dishwasher:  “Kenmore” Model #13803, Stainless Steel or approved equal

- Microwave/Hood Combo:  “Kenmore”  Model # 80333, Stainless Steel or approved equal

- Microwave must be vented outside using heavy gauge galvanized metal

Laundry Appliances

- Washer: top load: “Kenmore” #26-22102000, white or approved equal

- Dryer:  Gas, “Kenmore” #26-7120200, white or approved equal

Home Hardware

Window Blinds

- Style: Bali Cut-to-Size or Blinds.com Economy Light Filtering Double Cell, Cellular Shade in white/cotton, cordless

Shower rods (no tension)

- Style:  Cured

- Color:  Brushed Nickel

Mirrors (1/4” only)

- Size:  to match vanities

- Style: ¼” only frameless beveled

- Location:  Over vanities

Mailbox

- Style: Classic

- Color:  White  with house numbers

- Location: Mounted at front  porch exterior wall to meet USPS requirements or 4”x4” post with mailbox and numbers at street right-a-way

Radon Mitigation System:

Furnish and install a new active radon gas mitigation system complete. Furnish all required permits and facilitate all required inspections and approvals. Ensure proper combustion air to fuel burning appliances to prevent back draft from occurring. All work to be done by a National Environmental Professional, per ASTM standards, Designation E-2121-03. All Penetrations to be comply with applicable building, mechanical, plumbing, and electrical, energy and fire prevention codes and standards and regulations of the City Of Lansing. Supply Ingham County Land Bank with a copy of permit and final approval documentations.

Recycling of all construction materials:

As we move towards a more circular environment, promote best practices in recycling and repurposing leftover construction material, by reducing the amount of material to landfills. This includes all trades. Structural, Electrical, Plumbing and HAVC

SHED WITH CONCRETE PAD:

Excavate to provide sufficient depth to install a 4”inch thick concrete pad on 4”inch Minimum compacted sand fill for new shed 13’ x 10’ feet. Provide slope to drain water away from the structure. Furnish and install a new gambrel style wood shed 8’ x 10’ foot, including new framing 7/16” inch OSB wall and roof sheathing,3/4” inch OSB floor sheathing (if shed trusses are used). Include new vinyl siding and trim to match new house siding and trim. Include new roofing materials and asphalt shingles to match new house roofing. Include a double 24” inch entry reinforced doors with pulls, stops and astragal on 8” inch side. Include all door hardware including strap hinges, hasp, and keyed padlock. Provide Ingham County Land Bank with key immediately upon installation of padlock. All work to be done to ensure proper structural integrity and meet all current codes and zoning requirements. Location: Rear yard area and facing orientation: Exterior facing toward house.

NOTE: Shed exterior and door trim to be constructed with 1” inch x rough sawn cedar, 1” inch x (Azek) or similar approved material.  Smart board or similar approved material (seal all sides) or aluminum wrapped boards. Exposed pine will not be allowed.

Additional Notes:

At the conclusion of Construction:

 All ducts are to be cleaned of any debris & all floor drains, sump crocks and clean-outs are to be cleaned of any debris

 Sewer line is to be cleaned out and video’ed from basement cleanout to the street connection and copy of video to be provided to ICLB upon completion.

NOTE: Structural, Electrical, Plumbing and HAVC contractors to make all connections per Ingham County Land Bank selection sheets and specifications. All work to be done per local codes. NOTE: New Basement and Bedroom to be include in all trades and items on construction selection sheet.